Technical Datasheet for TnH Sensor Prob (Metal)

The TnH (Temperature and Humidity) Sensor Module provides highly accurate environmental readings over a standard I2C interface. Designed for straightforward, local integration, this sensor is highly efficient, boasts low power consumption, and is exceptionally easy to deploy in custom hardware builds.

Hardware & Design

- Rugged Metal Probe: The internal temperature and humidity sensing elements are securely housed within a durable, protective metal casing. This robust physical design shields the sensor from physical damage and environmental wear, making it suitable for a wide variety of indoor or enclosed monitoring applications.

- I2C Communication: This module utilizes a simple, two-wire I2C communication protocol (SDA, SCL). This makes it the perfect choice for short-distance, localized data acquisition where direct connection to a master microcontroller (such as a Raspberry Pi, Arduino, or custom DAQ board) is required.

- Wiring Interface: The probe features a flexible, multi-core pigtail cable with exposed wire leads, allowing technicians to quickly terminate and solder the connections directly to their expansion boards or terminal blocks.

| Pin | Name | Color | Description |
| --- | --- | --- | --- |
| 1 | VDD | Red | Power Supply (3.3V/ 5V) |
| 2 | SDA | Yellow | I2C line SDA |
| 3 | SCL | Green | I2C line SCL |
| 4 | GND | Black | Common Ground |

Specifications

General –

Power				Input Supply – 3.3 & 5V DC

Storage Temperature		-20 - 70 C (-4 ~ 158F)

Storage Humidity			5 ~ 95 % RH, non – Condensing

Certifications

|  | Sensor Name | Temperature Measuring Range | Humidity Measuring range | AQI/ GAS/ Pressure | Measuring Accuracy | Supply (VDD) |
| --- | --- | --- | --- | --- | --- | --- |
| ☐ | AHT20 | -20 ~ 85°C | 0~100% RH. | N/A | 0.3°C, 2.0 %RH | 3.3V/ 5V |
| ☐ | AHT21 | -40 ~ 120°C | 0~100% RH. | N/A | 0.3°C, 2.0 %RH | 3.3V/ 5V |
| ☐ | SHT40 | -40 ~ 125°C | 0~100% RH. | N/A | 0.2°C, 1.8 %RH | 3.3V |
| ☐ | SHT45 | -40 ~ 125°C | 0~100% RH. | N/A | 0.1°C, 1.0 %RH | 3.3V |
| ☐ | BME280 | -40 ~ 85°C | 0~100% RH. | 300 - 1100 hPa | 1.0°C, 3.0 %RH, 1.0 %hPa | 3.3V |
| ☐ | BME680 | -40 ~ 85°C | 0~100% RH. | 300 - 1100 hPa & 0 – 500 IAQ | 1.0°C, 3.0 %RH, 0.6 %hPa, 15 %IAQ | 3.3 V |
| ☐ | BME688 | -40 ~ 85°C | 0~100% RH. | 300 - 1100 hPa & 0 – 500 IAQ | 0.5°C, 3.0 %RH, 0.6 %hPa, 15 %IAQ | 3.3 V |
| ☐ | BME690 | -40 ~ 85°C | 0~100% RH. | 300 - 1100 hPa & 0 – 500 IAQ | 0.5°C, 3.0 %RH, 0.5 %hPa., 15 %IAQ | 3.3 V |

Contact us: -

Augmatic Technologies Pvt. Ltd.,

Plot no 6, Shah Industrial Estate II,

Kotambi,

Vadodara – 391510.

Email – Sales@wittelb.com

Support: support@wittelb.raiseaticket.com  / +91 94038 92805