LUMINOUS INTENSITY SENSOR

1. Product Overview

WIN-SN-LUX-200K-AOC-AOV is a compact, high-precision Light intensity sensor designed for applications requiring low power consumption and small form factors. It ensures reliable measurements over a wide range. Its Analog output with 4-20mA or 0-10v interface simplifies integration into small, embedded systems, making it ideal for IOT. The sensor's long-term stability ensures consistent performance in harsh environments, making it a cost-effective solution for long-term deployment.

Fig.1 - Product Image

• Luminance to Digital Converter ranging 1 LUX to 200000 LUX.

• Superior sensor performance.

• Operating range -20°C to 60°C and 0% to 70% RH for humidity

• Fully calibrated and processed digital, Analog Output

• Wide Input voltage options 12-24V DC / 230V AC (optional)

• Excellent long-term stability

| Measuring Parameter | Measuring Range | Operating Temperature |
| --- | --- | --- |
| Luminous intensity | 1-200000 LUX | -20°C ~ 60°C. |

2. Sensor Specifications

- Wide temperature range: 		−40 °C to +125 °C

- Full humidity range: 			0 % to 100 % RH

- Power supply DC:			12-24V@1A.

- Product dimension: 			100*100*60 mm.

- Certifications:

3. Power Supply Connection

DC Connection

- Use 12-24V@ 1A, Power supply.

- Don’t make loose connection.

Recommended Cable Electrical Characteristics

22 AWG Cable 					Shielded and twisted pair should be used.

Tinned Copper					Recommended

Nominal Conductor DCR			14.7 ohm / 1000 ft

Nominal Capacitance				11 pf / feet (conductor to conductor)

High Frequency Non-Insertion Loss	0.5db / 100ft

4. Technical Specifications

Measured Parameters: 		lux

Output Type: 			Analog Output-10-bit or 12-bit resolution

Output Range: 			0 – 10V, 4-20mA (Jumper selectable).

Contact Information

Augmatic Technologies Pvt. Ltd.,

Plot no 6, Shah Industrial Estate II,

Kotambi, Vadodara – 391510.

Email – Sales@wittelb.com

Support E.: support@wittelb.raiseaticket.com  / M.: 9403892805