Technical Datasheet of WIN-PiHAT-4DD-2AIAO

1. Overview

The PiHAT 4DD-2AIAO is a versatile mixed-signal expansion board for Raspberry Pi 4 and CM4, offering comprehensive I/O capabilities for industrial automation and control. It provides precise signal measurement and robust load switching by combining 4 opto-isolated Digital Inputs, 4 relay-driven Digital Outputs, 2 high-resolution Analog Inputs, and 2 Analog Outputs. This ensures seamless data collection and actuation between external field devices and the Pi’s GPIO header for reliable operation in industrial environments.

This PiHAT features a scalable architecture supporting independent analog and digital channels. Its consolidated mixed-signal design allows maximum I/O flexibility while maintaining a compact and reliable solution for embedded sensing, load automation, and precise industrial control applications.

Specifications

| Category | 8AI PiHAT |
| --- | --- |
| Input Type | Digital: Opto-isolated Digital Inputs Analog: 12-bit ADC Resolution (Selectable 0-5V, 0-10V, 4-20mA) |
| Output Type | Digital: Relay-Isolated Digital Outputs (5A @ 250VAC/30VDC) Analog: 12-bit DAC Resolution (0-10V Range) |
| Channel Count | 12 Channels Total (4 DI, 4 DO, 2 AI, 2 AO) |
| Connectors | Terminal Blocks (3.81 mm Pluggable): • One 2x8-Pin (for all DI & DO channels) • One 2x4-Pin (for all AI & AO channels) • One 2-Pin (24V DC; powers PiHAT & Raspberry Pi simultaneously) |

General :–

Dimensions			56 mm L x 85 mm W

Power			Input Power – 24V DC

Operating Temperature	0 – 60 C (32 ~ 140F)

Storage Temperature	-20 - 70 C (-4 ~ 158F)

Storage Humidity		5 ~ 95 % RH, non – Condensing

Certifications

AI (Analog Inputs) :–

Channels			2

ADC Resolution		12-bit

Input Type			Single-Ended / Differential (Depending on config)

Voltage Input Range	0 ~ 5V DC, 0 ~ 10V DC

Current Input Range	0 ~ 20mA, 4 ~ 20mA

Input Impedance(V)	≥ 1 MΩ

Input Impedance(I)	~ 250 Ω (Typical for mA drop)

Accuracy			±0.1% to ±0.3% FSR (Full Scale Range)

Sample Rate		Programmable (e.g., 128 SPS to 3300 SPS)

AO (Analog Outputs) :-

Channels			2

DAC Resolution		12-bit

Output Type		Single-Ended Voltage Output

Voltage Output Range	0 ~ 10V DC

Max Output Current	35 mA (Short-Circuit limit), ~15 mA continuous

Accuracy			±0.1% to ±0.3% FSR (Full Scale Range)

Settling Time		6 µs (Typical)

DI (Digital Inputs) :–

Channels			4

Input Type			12V & 5V with Reference, 80V Absolute Max. @ 50mA

Sense Voltage		3.3 – 12V DC (Depends on the selected type of Input)

Digital Sensing Logic	Logic '0' = <1 VDC, Logic '1' = >3.3 VDC

Isolation			Optically isolated

Response Time		2 msec, Max 6 msec

DO (Digital Outputs) :-

Channels			4

Output Type		Relay-Isolated Digital Outputs (5V SPST-NO Industrial Relay)

Contact Form		1A (SPST-NO)

Contact Material		AgNi Alloy

Contact Capacity		5A @ 250VAC, 5A @ 30VDC

Coil Voltage			5VDC

Coil Power			0.12W

Insulation Resistance	≥ 100MΩ

Electrical Life		1x10⁵ operations

Mechanical Life		1x10⁷ operations

Operating Time		≤10 ms (max., at rated voltage)

Release Time		≤05 ms (max., at rated voltage)

Configuration :-

The PiHAT 4DD-2AIAO operates as a comprehensive mixed-signal expansion board, interfacing 4 Digital Inputs, 4 Digital Outputs, 2 Analog Inputs, and 2 Analog Outputs to the Raspberry Pi GPIO header. Data acquisition and load control are managed simultaneously via software running on the Raspberry Pi.

Analog Configuration (AI & AO): The 2 AI and 2 AO channels are managed over a galvanically isolated I2C communication bus utilizing the ISO1540DR isolator. When an external analog signal is applied to an input terminal, the onboard ADS1015 ADC converts it into a precise 12-bit digital value. Conversely, software-generated 12-bit digital commands are sent to the onboard MCP4728 DAC, which immediately converts them into continuous, proportional analog voltages (0-10V) to drive external equipment.

Digital Configuration (DI & DO): The digital channels bypass the I2C bus and interface directly with dedicated Raspberry Pi GPIO pins. The 4 DI channels use optocouplers to safely step-down external signals, allowing your software to read active states as standard logic levels. The 4 DO channels are controlled by driving their assigned GPIO pins HIGH to energize the onboard relays (closing the NO contact) or LOW to de-energize them.

The following are step by step guides for configuration:

- Connect the Board to the Raspberry Pi via header pins

- Connect the Module to 24V DC POWER SUPPLY.

| I/O | Signal Name | Pi GPIO | Header Pin Number | Description |
| --- | --- | --- | --- | --- |
| 2AI & 2AO | SDA | 02 | 03 | Isolated I2C data line for ADS1015 (AI) & MCP4728 (AO) |
|  | SCL | 03 | 05 |  |
| 4DI | DI1 | 05 | 29 | Direct GPIO monitoring for opto-isolated inputs |
|  | DI2 | 06 | 31 |  |
|  | DI3 | 16 | 36 |  |
|  | DI4 | 26 | 37 |  |
| 4DO | DO1 | 22 | 15 | Direct GPIO control for relay-driven outputs |
|  | DO2 | 23 | 16 |  |
|  | DO3 | 24 | 18 |  |
|  | DO4 | 25 | 22 |  |

Wiring Diagram & Terminal Configuration:-

AO1|GND|AO2|GND

AI1|GND|AI2|GND

VIN|VIN|DI1|DI2|DI3|DI4|IN.G|IN.G

C1 |NO1 |C2 |NO2 |C3 |NO3|C4|NO4

Analog Inputs (AI1 – AI2)

- Terminals: Connect the sensor's positive signal wire to the AIx terminal and the common/ground wire to the corresponding GND terminal.

- Configuration: Each AI channel features a dedicated hardware jumper. Place the jumper on the 5V position to read 0-5V signals, or the 10V position for 0-10V signals. Always ensure this setting strictly matches your sensor's maximum output to prevent data clipping.

Analog Outputs (AO1 – AO2)

- Terminals: Connect the receiving device's signal input to the AOx terminal and its ground reference to the GND terminal. The DAC provides a proportional 0-10V control signal across these pins.

Digital Outputs (DO1 – DO4)

- Terminals: Each relay channel utilizes a Cx (Common) and NOx (Normally Open) terminal pair.

- Wiring: For load switching, connect the load's power supply (Phase/Positive) to Cx, and connect the load itself to NOx. When the software drives the GPIO high, the relay energizes, closing the internal circuit between Cx and NOx.

Digital Inputs (DI1 – DI4)

- Terminals: Connect the switching signals to the DIx terminals. Depending on your switching logic, DI signals are measured with respect to either VIN (Voltage Input Reference) or IN.G (Input Ground).

- Configuration: Use the onboard DI_PULL jumper to set the pull state (High/Low) for the optocouplers.

- External Power Warning: If your application uses an external reference voltage source to drive the digital inputs, you must completely remove the associated internal jumpers (VCC_SEL and IN.G) to ensure safe electrical isolation.

Sample Code & Examples :-

Access the official Python example scripts for 4DI, 4DO, 2AI, and 2AO control and testing from our repository here:

- WIN-PiHAT-4DD-2AIAO

Hardware Interface & Connector Layout:

40 pin GPIO Headers

24V DC SPLY

2

1

40

39

DI PULL-UP & PULL- DOWN

VIN Voltage Reference

0-5/0-10V

JUMPER

| C1 | NO1 | C2 | NO2 | C3 | NO3 | C4 | NO4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VIN | VIN | DI1 | DI2 | DI3 | DI4 | IN.G | IN.G |

| AI1 | GND | AI2 | GND |
| --- | --- | --- | --- |
| AO1 | GND | AO2 | GND |

VIN Voltage Reference

Contact us: -

Augmatic Technologies Pvt. Ltd.,

Plot no 6, Shah Industrial Estate II,

Kotambi,

Vadodara – 391510.

Email – Sales@wittelb.com

Support: support@wittelb.raiseaticket.com  / +91 94038 92805