Technical Datasheet TnH Sensor Modules with AOV/AOC

The WIN-SN-TNH-AOC-AOV (Outdoor Edition) is a robust sensor housed in a weather-resistant louvered enclosure, designed to accurately measure Temperature and Humidity in outdoor environments.

TnH Sensor Module: -

The device outputs precise analog signals with selectable 10-bit or 12-bit resolution, supporting both 0-10V and 4-20mA ranges for easy integration with industrial systems. Powered by a standard 12-24V DC supply, it features onboard resettable fuses to protect against reverse polarity on both the power and analog signal terminals. This resilient design makes it ideal for greenhouse climate control and general environmental monitoring.

Specifications

General –

Dimensions			158mm L x 139 mm D

Power			Input Power – 12 - 24 V DC

Operating Temperature	0 – 60 C (32 ~ 140F)

Storage Temperature	-20 - 70 C (-4 ~ 158F)

Storage Humidity		5 ~ 95 % RH, non – Condensing

Certifications

Technical Specifications: -

Measured Parameters 	Temperature and Humidity

Output Type		Analog Output 10-bit / 12-bit resolution

Output Range		AOV 0-10V / AOC 4-20mA

|  | Sensor Name | Temperature Measuring Range | Humidity Measuring range | Operating Temperature | Measuring Accuracy |
| --- | --- | --- | --- | --- | --- |
| ☐ | AHT20 | -20°C ~ 85°C. | 0~100% RH. | -40°C ~ 120°C. | 0.3°C, 2.0 %RH |
| ☐ | AHT21 | -40°C ~ 120°C. | 0~100% RH. | -40°C ~ 120°C. | 0.3°C, 2.0 %RH |

Applications

- Outdoor air quality monitoring.

- Climate control in greenhouses.

- Environmental monitoring systems.

- Smart home system.

Precautions

- Read all instructions before using the sensor board.

- Keep the sensor board away from water or moisture to prevent damage.

- Do not disassemble or modify the sensor board.

- Use only the specified power sources and mentioned.

Wire diagram:

| ID | AO | Wire Colour | Parameter |
| --- | --- | --- | --- |
| 1 | AO1 | Brown | Temperature + |
| 2 |  | Black | Temperature - |
| 3 | AO2 | Yellow | Humidity + |
| 4 |  | White | Humidity - |

Wire diagram:

| Analog Output | Parameter | Range |
| --- | --- | --- |
| AO1 | Temperature | -40°C ~ 85°0043 |
| AO2 | Humidity | 0 ~ 130 g/m3 |

Contact us: -

Augmatic Technologies Pvt. Ltd.,

Plot no 6, Shah Industrial Estate II,

Kotambi,

Vadodara – 391510.

Email – Sales@wittelb.com

Support: support@wittelb.raiseaticket.com  / +91 94038 92805