Technical Datasheet 4G Gateway with 8DO & 8AI

Product Overview: -

The 4G Gateway is an advanced networking device designed to provide seamless connectivity and data management capabilities for various applications. With support for 4G LTE connectivity, cloud connectivity via MQTT protocol, and robust storage options, it offers a comprehensive solution for remote monitoring and control systems. Equipped with an integrated I/O card featuring 8 Digital Outputs (DO), 8 Digital inputs (DI) and 8 Analog Inputs (AI), alongside a Modbus RS485 port and micro SIM compatibility, it caters to a wide range of industrial and IoT applications. The intuitive web-based UI interface allows for easy configuration and management, making it an ideal choice for both professionals and enthusiasts.

Specifications

General:-

•4G LTE Connectivity: Utilizes 4G LTE technology for reliable and high-speed internet access. If 4G is not available, the system automatically switches to Wi-Fi. If both 4G and Wi-Fi are unavailable, the data is securely stored in the SD card for later upload.

•Cloud Connectivity (MQTT): Seamlessly connects to cloud platforms using the MQTT protocol for efficient data transmission and management.

•Event Storage: Offers on board event storage, capable of storing up to 50,000 events or more. Includes an external memory card slot for easy data retrieval in the event of 4G network issues.

•Modbus RS485 Port: Features 1 port of Modbus RS485 for integration with industrial control systems and sensors.

•GPS Location Data: Provides GPS location data for monitoring of remote equipment.

•Micro Sim Compatibility: Supports micro-Sim card for flexible and convenient connectivity option.

•Web-based UI Interface: Intuitive web-based user interface for easy configuration, monitoring, and management.

• Integrated I/O Channels: Features 8 Digital Outputs (DO) & 8 Digital inputs (DI) for localized control and 8 Analog Inputs (AI) for direct sensor monitoring, expanding the gateway's standalone automation capabilities.

•Dimensions: 110 mm L x 110 mm B x 50 mm H.

•Power: Input Power – 15-40 VDC, Typical – 12V DC @ 150mA.

•Operating Temperature: 0 – 60C (32 ~ 140F).

•Storage Temperature: 20 - 70C (-4 ~ 158F).

•Storage Humidity: 5 ~ 95 % RH, non – Condensing.

Configuration Settings: -

- Power up the module by connecting a 12-24 V DC supply.

- Insert the SIM card into the designated slot.

- Place the Micro SD card into its designated slot.

- Connect MODBUS RTU Multiple Slave devices (up to 31) to the A & B Terminal

Of Module.

- Access the module's configuration hotspot

(ID: Wittelb-4G-GATEWAY, PASS: 123456789).

- Default IP Address of Module is “192.168.1.100/cpp1”.

- Upon entering the provided IP address, a configuration  webpage

Pages will be accessible.

Page1:- TCP/IP Configuration:-

- Enter MAC ID of the Ethernet interface (6 hex bytes, e.g. 14:33:5C:77:62:48).

- Enter IP address for the Ethernet port (default: 192.168.1.254).

- Enter Subnet Mask (default: 255.255.255.0).

- Enter Gateway address (default: 192.168.1.1).

- Click Submit to save settings; the module applies to the new Ethernet configuration immediately.

Page2:- On second page you can set device Baud Rate (9600 / 19200 / 38400 /   57600 / 115200), Data Bits (5 / 6 / 7 / 8), Parity (None / Odd / Even), and Stop Bits (1 / 2) for RS485 Communication

Page3:-

      - Navigate to page 3 on the cloud platform.

      - Input the Host IP or Domain.

      - Enter the port Number.

      - Enter the topics for publishing and receiving messages.

      - Provide a username and password if required.

      - Allow a few seconds for the module to restart and establish a connection to MQTT.

Page4:-

- Go to page 4 on the Modbus Master interface.

- Input the name and slave address for the designated slave.

- Specify the register address from which you want to read slave data.

- Indicate the register end address for the slave.

- Enter the function code corresponding to the desired slave.

Page5:-

- Go to page 5 on the I/O Config interface.

- You can view real-time values of both analog and digital inputs on the baseboard.

- Additionally, you have the option to assign specific names to each input/output

(I/O) as needed.

Page6:-

- Navigate to page 6 on the Logical Operations interface.

- Here, you can configure logical AND and OR operations for each of the 4 relays.

- You have the flexibility to assign any two digital inputs to a relay and choose the desired logical operation.

- Additionally, you can assign an analog input to a relay and define minimum and maximum threshold values for relay operation.

- If you prefer not to use any logical operation, simply select the "OFF" option for

input one.

4G Gateway connection Diagram:

SD Card Slot

Sim Card Slot

Modbus [A-B]

RS485 & Polling

Led

ETHERNET PORT

Additional Features: -

Communication ports are isolated

Input power reverse polarity safety

ESD Safety IEC 61000-4-2,  30KV contact,  30KV air

EFT IEC 61000-4-4, 50A (5/50ms)

400V isolation.

Base Board with 8 AI port with Jumper settings for 10 bit / 12 bit and 5v / 10 v Input Sources.

DO1    DO2       DO3     DO4       DO5      DO6      DO7      DO8

AI & DI Jumper select

Jumper Setting AI

5V / 10v

Jumper Setting AI

10b / 12 b

IN8         IN7         IN6         IN5        IN4         IN3        IN2       IN1

Relay 10A

Power I/P 24 V DC

Contact us: -

Augmatic Technologies Pvt. Ltd.,

Plot no 6, Shah Industrial Estate II,

Kotambi,

Vadodara – 391510.

Email – Sales@wittelb.com

Support: support@wittelb.raiseaticket.com  / +91 94038 92805