Augmatic Technologies – Converter Module

Configurations Manual

WIN - UART TTL TO ETHERNET

Configuration Step for WIN - UART TTL TO ETHERNET Converter

- Connect 5V DC Supply to the Module.

- Connect your UART TTL device to the TX, RX, and GND terminals of the Module.

- Connect the Network Ethernet Cable to the Ethernet port of the module.

- Default IP Address of the Module is “192.168.1.254”.

- After connecting, the module is ready for UART to TCP/UDP conversion.

- Please enter “192.168.1.254” in your web browser. Make sure your device is in the same network as the Module.

- After entering the above IP Address, you will find a webpage which has 6 pages for Configurations.

Page Number 1 (System Info)

- On the first page, you will get the Current System Information of the module.

- SW Version, Microcontroller, EEPROM Health, Ethernet Chip, Ethernet Sockets, MAC Address, and IP Address are available on this First page.

- Current hardware diagnostics and active network parameters are printed directly on the page for easy monitoring.

- On the left Selection panel, you can navigate to other page information.

- System Info, UART Status, IP Settings, TCP/UDP Settings, UART Settings, and available Tools are placed on the Left Selection Panel.

Page Number 2 (UART Status)

- The second page is setup for live monitoring and discovery of your serial connection.

- Users can use this page to actively search for connected devices by clicking the 'UART Scan' button.

- Once the scan is initiated, the module will probe the serial bus and display any actively responding UART slave addresses (e.g., "0Ah Scanning...").

- Users should ensure the physical connections (TX, RX, and GND) are secure on the module before initiating a scan to successfully discover connected devices.

Page Number 3 (IP Settings)

- Third Page is setup for IP Settings Configuration.

- MAC Address, Static IP, Sub mask & Gateway can be set from this page.

- MAC Address randomization can be done from the third page by clicking the 'Randomize' button.

- Users must click 'Save & Apply' to confirm and save any changes made to the network parameters.

Page Number 4 (TCP/UDP Settings)

- On the fourth page, TCP/UDP configuration can be applied by the user.

- UART TCP Port, UART UDP Port, & WebUI Port can be set from this page.

- UART Mode selection can be done from the fourth page.

- UART Mode: 1. UART TCP/UDP | 2. UART over TCP/UDP

- UART TCP Idle Timeout (in seconds) can also be configured here.

Page Number 5 (UART Settings)

- Fifth page is setup for UART Settings Configuration.

- Baud Rate, Data Bits, Parity, and Stop Bits can be set from this page.

- Inter-frame Delay, Response Timeout, and Retry Attempts as well.

- Users must click 'Save & Apply' to confirm any changes made to the serial communication parameters. [note: it will take few seconds for values to change.]

Page Number 6 (Tools)

- The sixth page provides essential system maintenance tools for the gateway.

- It allows the user to perform a system 'Reboot' or 'Load Default Settings' to restore the factory static IP (192.168.1.254).

- Also, on the module you can short the header H3 (DS_RST) to reset to default settings.

5.0 V

3.3 V

RXD

TXD

GND

GND

DS_RST

Power LED

ETHERNET PORT

Contact us: -

Augmatic Technologies Pvt. Ltd.,

Plot no 6, Shah Industrial Estate II,

Kotambi,

Vadodara – 391510.

Email – Sales@wittelb.com