Technical Datasheet WIN-SN-BBT-6NT-M(Slave)

The 6-Channel NTC Sensor Node communicates via a robust RS485 Modbus network, seamlessly transmitting multi-point temperature data back to the central Master Display Unit. Designed for absolute simplicity in the field, this module operates as a strictly plug-and-play node—requiring zero local configuration

Sensor Node Module: -

The sensor nodes are housed in an industrial, DIN-rail mountable enclosure featuring easily accessible terminal blocks for the six NTC inputs, power, and communication lines.

To ensure uninterrupted operation in volatile industrial environments, the module's internal design incorporates advanced resettable fuses. This provides critical hardware-level safeguarding against accidental reverse polarity and overcurrent events on both the Power and Communication ports.

Specifications

General –

Dimensions			84 mm W x 108 mm L x 36 mm H

Power			Input Power – 12 - 24 VDC

Storage Temperature	-20°C to +80°C (-4°F to +176°F)

Storage Humidity		5 ~ 95 % RH, non – Condensing

Certifications

NTCs :–

Channels					6

Input Resolution				12 bits

No-load resistance at 25°C		10 000

Tolerances at 25°C				± 1%

Beta(25/85) Constant			3 435K ±1%

Operating temperature range		-40°C to +105°C

IP Rating					IP 67

Additional Features: -

All inputs  and communication ports isolated

Input power reverse polarity safety

ESD Safety IEC 61000-4-2,  30KV contact,  30KV air

EFT IEC 61000-4-4, 50A (5/50ms)

750V isolation.

CRC Error check.

Contact us: -

Augmatic Technologies Pvt. Ltd.,

Plot no 6, Shah Industrial Estate II,

Kotambi,

Vadodara – 391510.

Email – Sales@wittelb.com

Support: support@wittelb.raiseaticket.com  / +91 94038 92805